Library Includes: Each library gives specific capabilities:

- WiFi.h - Enables ESP32 to connect to wireless networks

- Firebase_ESP_Client.h - Allows communication with Google's Firebase cloud database

- DHT.h - Provides functions to read temperature/humidity from DHT22 sensor

- Wire.h - Implements I2C communication protocol for OLED display

- Adafruit_GFX.h & Adafruit_SSD1306.h - Graphics libraries for controlling OLED screen

Pin Definitions:

- MQ135_PIN 34 - Analog pin A2 (GPIO34) for air quality sensor

- MQ4_PIN 35 - Analog pin A3 (GPIO35) for methane gas sensor

- DHT_PIN 4 - Digital pin GPIO4 for temperature/humidity sensor

- BUZZER_PIN 13 - Digital pin GPIO13 for piezo buzzer

Object Creation:

- Adafruit_SSD1306 display() - Creates an object that represents the physical OLED screen

- DHT dht() - Creates an object to interface with DHT22 sensor

- FirebaseData fbdo - Object to handle data transfer to/from Firebase

- FirebaseAuth auth - Object for authentication with Firebase

- FirebaseConfig config - Object to store Firebase connection settings

Global Variables:

- These variables exist in global memory space and are accessible by ALL functions

- mq135_value, mq4_value store raw analog readings (0-4095 on ESP32)

- temperature, humidity store processed floating-point values

- alert_status is a boolean flag that indicates if any threshold is exceeded

Timing Mechanism:

- millis() returns the number of milliseconds since program started (overflows after 50 days)

- currentMillis - lastUpdate >= UPDATE_INTERVAL creates a non-blocking delay

- This allows the ESP32 to do other tasks while waiting, unlike delay() which freezes everything

Setup Sequence:

- Serial.begin(115200) - Starts serial communication at 115200 bits per second

- Functions are called in specific order because some depend on others

- Display first (user feedback), then sensors, then network connections

Main Loop Logic:

- Runs continuously at high speed (millions of cycles per second)

- Only executes the sensor reading block every 2000ms (2 seconds)

- This prevents overwhelming the sensors and cloud service with too many readings